Laboratory Exercise 11

Interrupts & IO programming

Assignment 1:

- Source code:

#------------------------------------------------------

# 		col 0x1 col 0x2 col 0x4 col 0x8

#

#row 0x1	0 	1 	2 	3

# 		0x11 	0x21 	0x41 	0x81

#

#row 0x2 	4 	5 	6	7

# 		0x12 	0x22 	0x42 	0x82

#

#row 0x4 	8 	9 	a 	b

# 		0x14 	0x24 	0x44 	0x84

#

#row 0x8 	c 	d 	e 	f

# 		0x18 	0x28 	0x48 	0x88

#

#------------------------------------------------------

# command row number of hexadecimal keyboard (bit 0 to 3)

# Eg. assign 0x1, to get key button 0,1,2,3

#     assign 0x2, to get key button 4,5,6,7

# NOTE must reassign value for this address before reading,

# eventhough you only want to scan 1 row

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

# receive row and column of the key pressed, 0 if not key pressed

# Eg. equal 0x11, means that key button 0 pressed.

# Eg. equal 0x28, means that key button D pressed.

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.text

main:		li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t2, OUT_ADRESS_HEXA_KEYBOARD

li $t3, 0x08	# check row 4 with key C, D, E, F

polling:	sb $t3, 0($t1)	# must reassign expected row

lb $a0, 0($t2)	# read scan code of key button

print:	li $v0, 34	# print integer (hexa)

syscall

sleep:	li $a0, 100	# sleep 100ms

li $v0, 32

syscall

back_to_polling: j polling	# continue polling

- Kết quả:

Assignment 2:

- Source code:

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.data

Message: .asciiz "Oh my god. Someone's presed a button.\n"

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# MAIN Procedure

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.text

main:

#---------------------------------------------------------

# Enable interrupts you expect

#---------------------------------------------------------

# Enable the interrupt of Keyboard matrix 4x4 of Digital Lab Sim

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t3, 0x80 # bit 7 of = 1 to enable interrupt

sb    $t3,   0($t1)

#---------------------------------------------------------

# No-end loop, main program, to demo the effective of interrupt

#---------------------------------------------------------

Loop: 	nop

nop

nop

nop

b       Loop	# Wait for interrupt

end_main:

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~ ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# GENERAL INTERRUPT SERVED ROUTINE for all interrupts

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.ktext 0x80000180

#--------------------------------------------------------

# Processing

#--------------------------------------------------------

IntSR: 	addi $v0, $zero, 4 	# show message

la $a0, Message

syscall

#--------------------------------------------------------

# Evaluate the return address of main routine

#epc <=epc+4

#--------------------------------------------------------

next_pc:mfc0 $at, $14	# $at <=  Coproc0.$14 = Coproc0.epc

addi $at, $at, 4	# $at = $at + 4   (next instruction)

mtc0 $at, $14	# Coproc0.$14 = Coproc0.epc <= $at

return: eret		# Return from exception

- Kết quả:

Assignment 3:

- Source code:

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADRESS_HEXA_KEYBOARD 0xFFFF0014

.data

Message: .asciiz "Key scan code "

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# MAIN Procedure

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.text

main:

#---------------------------------------------------------

# Enable interrupts you expect

#---------------------------------------------------------

# Enable the interrupt of Keyboard matrix 4x4 of Digital Lab Sim

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t3, 0x80 # bit 7 = 1 to enable

sb $t3, 0($t1)

#---------------------------------------------------------

# Loop an print sequence numbers

#---------------------------------------------------------

xor $s0,$s0,$s0		#count=$s0=0

Loop: 		addi $s0, $s0, 1	# count = count + 1

prn_seq:	addi $v0,$zero,1

add $a0,$s0,$zero	# print auto sequence number

syscall

prn_eol:	addi $v0,$zero,11

li $a0,'\n' 		# print endofline

syscall

sleep: 		addi $v0,$zero,32

li $a0,300		# sleep 300 ms

syscall

nop			# WARNING: nop is mandatory here.

b Loop			# Loop

end_main:

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# GENERAL INTERRUPT SERVED ROUTINE for all interrupts

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.ktext 0x80000180

#-------------------------------------------------------

# SAVE the current REG FILE to stack

#-------------------------------------------------------

IntSR:

addi $sp,$sp,4	# Save $ra  because we may change it later

sw $ra,0($sp)

addi $sp,$sp,4	# Save $ra  because we may change it later

sw $at,0($sp)

addi $sp,$sp,4	# Save $ra  because we may change it later

sw $v0,0($sp)

addi $sp,$sp,4	# Save $a0, because we may change it later

sw $a0,0($sp)

addi $sp,$sp,4	# Save $t1, because we may change it later

sw $t1,0($sp)

addi $sp,$sp,4	# Save $t3, because we may change it later

sw $t3,0($sp)

#--------------------------------------------------------

# Processing

#--------------------------------------------------------

prn_msg:addi $v0, $zero, 4

la $a0, Message

syscall

get_cod:li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t3, 0x88 # check row 4 and re-enable bit 7

sb $t3, 0($t1) # must reassign expected row

li $t1, OUT_ADRESS_HEXA_KEYBOARD

lb $a0, 0($t1)

prn_cod:li $v0,34

syscall

li $v0,11

li $a0, '\n'	# print endofline syscall

#--------------------------------------------------------

# Evaluate the return address of main routine

#epc <=epc+4

#--------------------------------------------------------

next_pc:mfc0 $at, $14 		# $at <=  Coproc0.$14 = Coproc0.epc

addi $at, $at, 4	# $at = $at + 4   (next instruction)

mtc0 $at, $14		# Coproc0.$14 = Coproc0.epc <= $at

#--------------------------------------------------------

# RESTORE the REG FILE from STACK

#--------------------------------------------------------

restore:lw $t3, 0($sp)	# Restore the registers from stack

addi $sp,$sp,-4

lw $t1, 0($sp)	# Restore the registers from stack

addi $sp,$sp,-4

lw $a0, 0($sp)	# Restore the registers from stack

addi $sp,$sp,-4

lw $v0, 0($sp)	# Restore the registers from stack

addi $sp,$sp,-4

lw $ra, 0($sp)	# Restore the registers from stack

addi $sp,$sp,-4

return: eret		 # Return from exception

- Kết quả:

Assignment 4:

- Source code:

.eqv IN_ADRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv  COUNTER         0xFFFF0013	# Time Counter

.eqv  MASK_CAUSE_COUNTER   0x00000400	 # Bit 10: Counter interrupt

.eqv  MASK_CAUSE_KEYMATRIX 0x00000800	 # Bit 11: Key matrix interrupt

.data

msg_keypress: .asciiz "Someone has pressed a key!\n"

msg_counter: .asciiz "Time inteval!\n"

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# MAIN Procedure

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.text

main:

#---------------------------------------------------------

# Enable interrupts you expect

#---------------------------------------------------------

# Enable the interrupt of Keyboard matrix 4x4 of Digital Lab Sim

li $t1, IN_ADRESS_HEXA_KEYBOARD

li $t3, 0x80 	# bit 7 = 1 to enable

sb $t3, 0($t1)

# Enable the interrupt of TimeCounter of Digital Lab Sim

li $t1, COUNTER

sb $t1, 0($t1)

#---------------------------------------------------------

# Loop an print sequence numbers

#---------------------------------------------------------

Loop: 	nop

nop

nop

sleep: 	addi $v0,$zero,32 # BUG: must sleep to wait for Time Counter

li $a0,200 	# sleep 300 ms

syscall

nop		# WARNING: nop is mandatory here.

b Loop

end_main:

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# GENERAL INTERRUPT SERVED ROUTINE for all interrupts

#~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.ktext 0x80000180

IntSR: 	#--------------------------------------------------------

# Temporary disable interrupt

#--------------------------------------------------------

dis_int:li $t1, COUNTER  # BUG: must disable with Time Counter

sb $zero, 0($t1)

# no need to disable keyboard matrix interrupt

#--------------------------------------------------------

# Processing

#--------------------------------------------------------

get_caus:mfc0 $t1, $13 		# $t1 = Coproc0.cause

IsCount:li $t2, MASK_CAUSE_COUNTER# if Cause value confirm Counter..

and $at, $t1,$t2

beq $at,$t2, Counter_Intr

IsKeyMa:li $t2, MASK_CAUSE_KEYMATRIX # if Cause value confirm Key..

and $at, $t1,$t2

beq $at,$t2, Keymatrix_Intr

others: j end_process 		# other cases

Keymatrix_Intr: li $v0, 4 	# Processing Key Matrix Interrupt

la $a0, msg_keypress

syscall

j end_process

Counter_Intr: li $v0, 4 	# Processing Counter Interrupt

la $a0, msg_counter

syscall

j end_process

end_process:

mtc0 $zero, $13 	# Must clear cause reg

en_int:	#--------------------------------------------------------

# Re-enable interrupt

#--------------------------------------------------------

li $t1, COUNTER

sb $t1, 0($t1)

#--------------------------------------------------------

# Evaluate the return address of main routine

#epc <=epc+4

#--------------------------------------------------------

next_pc:mfc0 $at, $14 		# $at <=  Coproc0.$14 = Coproc0.epc

addi $at, $at, 4	# $at = $at + 4   (next instruction)

mtc0 $at, $14		# Coproc0.$14 = Coproc0.epc <= $at

return: eret 			# Return from exception

- Kết quả:

Assignment 5:

- Source code:

.eqv KEY_CODE 0xFFFF0004 	# ASCII code from keyboard, 1 byte

.eqv KEY_READY 0xFFFF0000 	# =1 if has a new keycode ?

# Auto clear after lw

.eqv DISPLAY_CODE 0xFFFF000C 	# ASCII code to show, 1 byte

.eqv DISPLAY_READY 0xFFFF0008 	# =1 if the display has already to do

# Auto clear after sw

.eqv MASK_CAUSE_KEYBOARD 0x0000034 # Keyboard Cause

.text

li   $k0,  KEY_CODE

li   $k1,  KEY_READY

li   $s0, DISPLAY_CODE

li   $s1, DISPLAY_READY

loop:        	nop

WaitForKey:  	lw $t1, 0($k1)			# $t1 = [$k1] = KEY_READY

beq $t1, $zero, WaitForKey 	# if $t1 == 0 then Polling

MakeIntR: 	teqi $t1, 1 		# if $t0 = 1 then raise an Interrupt

j loop

#---------------------------------------------------------------

# Interrupt subroutine

#---------------------------------------------------------------

.ktext 0x80000180

get_caus:	mfc0  $t1, $13              # $t1 = Coproc0.cause

IsCount:	li $t2, MASK_CAUSE_KEYBOARD# if Cause value confirm Keyboard..

and $at, $t1,$t2

beq $at,$t2, Counter_Keyboard

j end_process

Counter_Keyboard:

ReadKey: 	lw $t0, 0($k0) 		# $t0 = [$k0] = KEY_CODE

WaitForDis:	lw $t2, 0($s1) 		# $t2 = [$s1] = DISPLAY_READY

beq $t2, $zero, WaitForDis # if $t2 == 0 then Polling

Encrypt:	addi $t0, $t0, 1	# change input key

ShowKey:	sw $t0, 0($s0)		# show key

nop

end_process:

next_pc:     	mfc0 $at, $14		# $at <= Coproc0.$14 = Coproc0.epc

addi $at, $at, 4	# $at = $at + 4 (next instruction)

mtc0 $at, $14	# Coproc0.$14 = Coproc0.epc <= $at

return:		eret			# Return from exception

- Kết quả: